# SmartTar — Reference Manual

Version 2.34

MicroDiseño Ltda.

Copyright © 1993–2003. All rights reserved.

# Product Overview

SmartTar is a real-time telephone tariff management system designed for public telephone booth centers (cabinas telefónicas). It runs under MS-DOS 5.0 in 286 protected mode via the Pharlap DOS extender, and manages the complete call lifecycle from off-hook detection through tariff calculation, receipt printing, and transaction archival.

SmartTar was developed by MicroDiseño Ltda. (Gonzalo Contento Castañ) starting in July 1993. Version 2.34 supports up to 32 telephone booths across 4 hardware clusters of 8 booths each.

## Key Capabilities

- Real-time call metering. Monitors up to 32 telephone booths simultaneously. Each booth is polled in real time for hook state, DTMF digits, and answer signals.

- Automatic call classification. Resolves the dialed number prefix against a configurable numbering plan to determine the call type: local, national long-distance (DDN), international (DDI), border zone, cellular, or special service.

- Tariff engine. Applies time-of-day and day-type schedules to compute the call cost. Supports up to 16 DDN tariff bands and up to 20 DDI tariff bands, with up to 6 schedule slots and 3 day types (normal, Saturday, holiday).

- Holiday calendar. Configurable per year; used by the tariff scheduler to switch to holiday tariff rates on designated dates.

- Receipt printing. Issues numbered, tax-inclusive (IVA) receipts through a plug-in printer driver system. Supports 18-, 28-, 40-, and 80-column printers including thermal models.

- Transaction database. Indexed file store (RX.DAT / RX.IDX) with duplicate-call detection and multi-turn archiving.

- Prepaid magnetic card support. Up to 4 card denominations configurable.

- Modem integration. Remote reporting and configuration synchronization via the SmartTar Communicator (STC) companion application.

- External booth display. Optional serial customer-facing LED display per cluster.

- Statistics module. Daily, weekly, monthly, and annual call and revenue summaries.

- Hardware copy protection. Parallel-port EEPROM dongle (bypassable only in demo builds).

## Product Variants

SmartTar Standard. Supports public telephone booths only. Receipt database stores calls by booth number.

SmartTar Pro. Extends the standard edition with PBX extension line billing. Each booth can be designated as either a public booth or an internal extension. Extensions receive a configurable discount percentage and may be billed for installation and monthly line costs. A separate receipt database (RXX.DAT / RXX.IDX) records extension calls.

# System Requirements

| Component | Requirement |
| --- | --- |
| Operating System | MS-DOS 5.0 or later |
| Processor | Intel 80286 or compatible |
| Conventional memory | 640 KB (required by DOS extender loader) |
| Extended memory | Managed by Pharlap 286 DOS extender |
| Display | EGA or VGA graphics adapter |
| Input | Keyboard; mouse optional |
| Printer | Serial (COM) or parallel (LPT); 18–80 column |
| Hardware telephone interface | STB-x cluster board (1–4 clusters of 8 booths each) |
| Copy protection | Parallel-port EEPROM dongle (production builds) |
| Modem (optional) | Hayes-compatible; any baud rate supported by COM port |

# Architecture Overview

SmartTar is a single-process, event-driven application structured in layered subsystems.

## Subsystem Layers

+-----------------------------------------------------+
|  Zinc UI layer  (menus, dialogs, toolbar, views)    |
+------------------+----------------------------------+
|  Controller      |  Real-time engine (rt_eng)       |
|  (control.cpp)   |  -- per-booth call state machine |
|                  |  -- ISR-driven timing (rt_isr)   |
+------------------+----------------------------------+
|  Tariff engine   |  Database engine (db_eng)        |
|  (ph_eng)        |  -- RX.DAT / RX.IDX              |
|                  |  -- receipt lifecycle            |
+------------------+----------------------------------+
|  Serial driver  Print spooler  Dongle  Modem        |
|  Config (cfg)   Logger (log)   Screen saver         |
+-----------------------------------------------------+
          Pharlap 286 DOS Extender
          MS-DOS 5.0 / PC hardware

## Core Components

- CONTROLLER (control.cpp). Central coordinator. Owns references to all major subsystems. Drives the Zinc event loop and dispatches events to subsystems. Manages receipt queues (Receipts, PRNReceipts).

- RT_ENGINE (rt_eng.cpp). Real-time engine. Scans hardware ports for booth state changes at regular intervals. Maintains a BoothCluster array with the per-booth call state machine. Driven by a software timer ISR (rt_isr.cpp).

- PH_ENGINE (ph_eng.cpp). Telephony and tariff engine. Loads PH_INFO.DAT containing local, DDN, and DDI place tables, tariff bands, and schedule tables. Provides call classification and cost calculation.

- DB_ENGINE (db_eng.cpp). Transaction database engine. Wraps DB_STORAGE (indexed file access to RX.DAT/RX.IDX) and DB_STATISTICS (running totals). In SmartTar Pro builds also manages RXX.DAT/RXX.IDX.

- CFG (cfg.cpp). Configuration manager. Reads ST.CFG (binary, encrypted) and ST.INI (text). Writes changes back to both files on demand.

- SPOOLER (spooler.cpp). Print spooler. Queues print jobs and dispatches them to the active printer driver DLL. Supports single or double printer configuration.

- SERIAL (serial.cpp). Serial port driver. Used by the optional booth display (BoothDisplay) and the modem subsystem.

- STM2 (stm2.cpp). Non-volatile state manager. Stores critical values (receipt counters, last shutdown time) in the EEPROM dongle. Detects and recovers from abnormal shutdowns.

- Log (log.cpp). Event logger. Appends structured log entries to ST.LOG for startup, shutdown, access denial, and other system events.

## Hardware Interface

Each STB-x hardware cluster occupies a 16-byte I/O port block starting at APP_PORT_BASE. The port layout within a cluster is:

- PO_GENERAL (0x00): General output port (relays, cash drawer).

- PO_DTMF_FLAGS: DTMF digit availability flags. All bits set (0xFF) indicates no STB board present.

ACTIVE_CLUSTERS is determined at startup by reading the DTMF flag ports of all configured clusters. Any cluster returning 0xFF is considered absent.

# Runtime Files Reference

| File | Purpose |
| --- | --- |
| ST.EXE | Main application executable (Pharlap protected-mode) |
| ST.CFG | System configuration (binary, encrypted). Written by SETUP utility. |
| ST.INI | Operator-editable runtime settings (plain text). Subset of ST.CFG. |
| RES.DAT | Zinc UI resource file (windows, dialogs, bitmaps, strings). |
| HELP.DAT | In-application help database (generated from DOC.TXT). |
| PH_INFO.DAT | Compiled telephony database (binary). Generated from DDI.INF + DDN.INF + LOCAL.INF. |
| DDI.INF | International telephony numbering plan and tariff source (text). |
| DDN.INF | National long-distance telephony numbering plan and tariff source (text). |
| LOCAL.INF | Local hometown numbering plan source (text). |
| RX.DAT | Transaction record file. Sequential binary records. |
| RX.IDX | Transaction index file. |
| RX.STA | Statistics snapshot file. |
| RXX.DAT | Extension call records (SmartTar Pro only). |
| RXX.IDX | Extension call index (SmartTar Pro only). |
| ST.LOG | System event log (text, appended). |
| PR_*.DLL | Printer driver DLLs (Pharlap dynamic libraries). |

# Configuration Reference

Configuration is stored in two files: ST.CFG (binary, encrypted, written by SETUP) and ST.INI (plain text, editable by the application at runtime). When the application saves configuration from the GUI, it writes both files.

## [Sistema] — System Parameters

| Parameter | Type | Description |
| --- | --- | --- |
| COUNTRY | String (32) | Country name where the system is installed. |
| CURRENCY | String (16) | Currency symbol (e.g., ,US, S/.). |
| CITY | String | City name where the center is located. |
| COMPANY | String | Company name of the telephone center operator. |
| OPERATOR_NAME | String | Name of the telephony service provider. |
| ID | String (32) | Tax identification number (NIT, CC, etc.). |
| TAX_NAME | String (32) | Name of the applicable tax (e.g., IVA). |
| TAX_PERCENT | Double | Tax percentage applied to standard calls. |
| DDN_TAX | Double | Tax percentage applied to national long-distance calls. |
| DDI_TAX | Double | Tax percentage applied to international calls. |
| CLUSTERS | Integer | Number of hardware clusters configured (1–4). |
| FORM | Enum | Printer form type. See Section 9. |
| P_PORT | String | Printer port type: lpt or com. |
| LPT | Integer | Parallel port number (1 or 2) when P_PORT=lpt. |
| COM | String | Serial port settings when P_PORT=com (port, baud, bits, parity, stop). |
| DOUBLE_PRN | Boolean | Enable dual-printer operation. |
| P_FOOTER | String (144) | Footer text printed at the bottom of each receipt. |
| P_FOOTER1 | String (64) | Receipt footer line 1. |
| P_FOOTER2 | String (64) | Receipt footer line 2. |
| HEADER_LINE | String (256) | Full header line printed at the top of receipts. |
| HEADER_LINE1 | String (64) | Receipt header line 1. |
| HEADER_LINE2 | String (64) | Receipt header line 2. |
| HEADER_LINE3 | String (64) | Receipt header line 3. |
| HEADER_LINE4 | String (64) | Receipt header line 4 (may include %s or %d format). |
| HEADER_PRINT_TAXNAME | Boolean | Print tax name on receipt header. |
| HEADER_PRINT_RECNO | Boolean | Print receipt number on receipt header. |
| SS_TIME | Integer | Screen saver timeout (minutes). |

## [Aplicacion] — Application Parameters

| Parameter | Type | Description |
| --- | --- | --- |
| N_RECEIPT | Long | Current receipt counter (0–999999). Preserved in EEPROM. |
| M_ROUND | Double | Rounding granularity for invoice totals. |
| VIEW_DECIMALS | Integer | Decimal places shown for call cost in the main view. |
| MIN_NAL | Integer | Minimum billed minutes for national calls. |
| MIN_INTER | Integer | Minimum billed minutes for international calls. |
| MIN_USA | Integer | Minimum billed minutes for calls to the USA. |
| MIN_BORDER | Integer | Minimum billed minutes for border zone calls. |
| MIN_CELLULAR | Integer | Minimum billed minutes for cellular calls. |
| CEIL_NAL | Double | Minute ceiling fraction for national calls (1.0 = whole minute, 0.5 = half). |
| CEIL_INTER | Double | Minute ceiling fraction for international calls. |
| CEIL_USA | Double | Minute ceiling fraction for USA calls. |
| CEIL_BORDER | Double | Minute ceiling fraction for border zone calls. |
| CEIL_CELLULAR | Double | Minute ceiling fraction for cellular calls. |
| APPLY_DDN_SCHEDULE | Boolean | Apply reduced-rate schedule for DDN calls. |
| APPLY_DDI_SCHEDULE | Boolean | Apply reduced-rate schedule for DDI calls. |
| GENERATE_PREPAID_RECEIPT | Boolean | Automatically generate a prepaid receipt. |
| DOUBLE_PREPAID_RECEIPT | Boolean | Print two copies of prepaid receipts. |
| MULTIPLE_PREPAID_CALLS | Boolean | Allow multiple calls within a single prepaid session. |
| CALL_ACTUAL_COST | Boolean | Show actual computed call cost rather than prepaid balance consumed. |
| MAX_COM_ERR | Integer | Maximum communication errors before alarm. |
| MAX_DIAL_ERR | Integer | Maximum dialing errors before alarm. |

## [Telefonia] — Telephony Timing Parameters

All timing values marked (ms) are in milliseconds.

| Parameter | Unit | Description |
| --- | --- | --- |
| T_ON_HOOK | ms | On-hook detection time. |
| T_OFF_HOOK | ms | Off-hook detection time. |
| T_BREAK | ms | Circuit break time for pulse (decadic) dialing. |
| T_MAKE | ms | Circuit make time for pulse dialing. |
| T_INTERDIG | ms | Inter-digit gap for pulse dialing. |
| T_DTMF_FLAG | ms | DTMF digit flag settling time. |
| T_DTMF_INTERDIG | ms | DTMF inter-digit gap. |
| T_ANSWER | ms | Answer timeout. |
| T_TALK | ms | Grace period after answer before billing starts. |
| T_BIAS | ms | Polarity inversion signal duration (when ASIGNAL = S_BIAS). |
| T_DIAL | ms | Maximum time allowed for dial sequence completion. |
| T_COM | ms | Communication start timeout (when ASIGNAL = S_TIME). |
| T_LOCK | ticks | Post-hangup lock time before booth is cleared in the view. |
| T_INTER_RING | ms | Maximum gap between rings; longer gap treated as on-hook. |
| ASIGNAL | Enum | Answer signal detection method. |

Answer signal methods (ASIGNAL):

- S_BIAS (polarity inversion). Answer detected by polarity inversion. T_BIAS specifies whether the inversion is permanent or lasts 150 ms.

- S_TIME (time). Answer assumed after T_COM seconds from off-hook.

- S_THREAD (C-wire). Answer detected by C-wire earth or −48 V signal.

- S_TONE (tone). Answer detected by tone recognition.

## [Marcacion] — Dialing Parameters

| Parameter | Type | Description |
| --- | --- | --- |
| ACCESS_LEVELS | Integer | Number of access levels (2 or 3) in the dialing plan. |
| ACCESS | Integer | Trunk access digit (e.g., 0 or 9). |
| INTER_ACCESS | Integer | Second access digit for international calls. |
| CELLULAR_ACCESS | Integer | Second access digit for cellular calls. |
| BORDER_ACCESS | Integer | Second access digit for border zone calls. |
| SPECIAL_ACCESS | Integer | Access digit for special service calls. |
| OPERATOR_ACCESS | Integer | Access code for operator-assisted calls. |
| EDA_ACCESS | Integer | Access digit for EDA operator calls. |
| LOCAL_DIGITS | Integer | Number of digits expected for local calls. |
| NAL_DIGITS | Integer | Number of digits expected for national calls. |
| INTER_DIGITS | Integer | Number of digits expected for international calls. |
| BORDER_DIGITS | Integer | Number of digits for border zone calls. |
| CELLULAR_DIGITS | Integer | Number of digits for cellular calls. |
| SPECIAL_DIGITS | Integer | Number of digits for special service calls. |
| IGNORE_EXTRA_DIGITS | Boolean | Ignore digits beyond the configured count. |

## [Extensiones] — Extension Parameters (SmartTar Pro)

| Parameter | Type | Description |
| --- | --- | --- |
| E_DISCOUNT | Double | Discount percentage applied to extension call costs. |
| E_INSTALL_COST | Double | One-time installation charge for extension lines. |
| E_LINE_COST | Double | Monthly line rental cost for extension lines. |
| E_FIRST_EXT | Integer | First extension number assigned. |
| E_SHOW_PHONE | Boolean | Show dialed phone number in the view for extension calls. |
| E_MIN_AVAILABLE | Double | Minimum account balance required for an extension call. |
| E_APPLY_ROUND | Boolean | Apply rounding to extension call totals. |

## [Modem] — Modem Parameters

| Parameter | Type | Description |
| --- | --- | --- |
| MODEM_COM | Integer | COM port number used by the modem. |
| MODEM_IRQ | Integer | IRQ line for the modem COM port. |
| MODEM_BAUDS | Long | Baud rate for modem communication. |
| MODEM_DIAL | Integer | Dial type: tone or pulse. |
| MODEM_SPEAKER | Integer | Modem speaker mode. |
| MODEM_PHONE | String | Last successfully connected remote phone number. |
| MODEM_ACKTIME | Integer | Acknowledge timeout for STC protocol messages. |
| MODEM_MAXTIME | Long | Maximum connection time. |

## [Display] — Booth Display Parameters

| Parameter | Type | Description |
| --- | --- | --- |
| DISPLAY_ENABLE | Boolean | Enable the customer-facing booth display. |
| DISPLAY_COM | Integer | COM port connected to the display unit. |
| DISPLAY_BAUDS | Integer | Baud rate for display communication. |
| DISPLAY_DEFAULT_MESSAGE | String (64) | Message shown when no call is in progress. |

# Telephony Database Reference

The telephony database is compiled from three source files by the SETUP utility into PH_INFO.DAT. It contains the numbering plan, tariff bands, and schedules for local, national long-distance (DDN), and international (DDI) calls.

## Source Files

- LOCAL.INF. Defines the local hometown numbering plan. Each entry maps a dialed prefix to a destination city name and call category. The sample file covers Antioquia, Colombia (434 entries).

- DDN.INF. Defines national long-distance (domestic) numbering plan and base tariff bands. Entries identify area codes, destination city names, and associated tariff indices.

- DDI.INF. Defines international telephony information. Entries identify country codes, country names, and associated DDI tariff indices.

## Tariff Structure

- DDN tariffs. Up to 16 tariff bands, each defining a per-minute rate for a destination group. Rates vary by time-of-day schedule (normal, reduced, super-reduced). Up to 6 schedule time slots per day type.

- DDI tariffs. Up to 20 tariff bands covering international destinations. Each band defines rates for normal, reduced, and super-reduced periods.

- Local tariffs. Defined per local zone using a configurable default tariff list (DEF_TARIFFS). Up to 9 named tariff levels (A1–A9).

- Day types. The scheduler distinguishes three day types: normal weekday, Saturday, and holiday. The holiday calendar is checked at call start to select the appropriate tariff set.

- Schedules. DDN and DDI schedules define up to 6 time windows per day type. APPLY_DDN_SCHEDULE and APPLY_DDI_SCHEDULE enable/disable schedule switching.

## Locked Numbers

PH_INFO.DAT also stores a list of locked (blocked) telephone numbers. Calls to locked numbers are intercepted before dialing completes. This is used to block premium-rate or unauthorized destinations.

# Call Lifecycle

Each telephone booth in RT_ENGINE maintains an independent call state machine.

## State Machine

| State | Description |
| --- | --- |
| IDLE | Booth is on-hook and available. |
| OFF_HOOK | Handset lifted. DTMF/pulse digit accumulation begins. Dial timeout timer (T_DIAL) starts. |
| DIALING | Digits are being accumulated. Prefix parser matches against the numbering plan. Locked numbers are checked. |
| RINGING / WAITING | Dialing complete; waiting for answer signal. Answer timeout timer (T_ANSWER) runs. |
| CONNECTED | Answer signal detected. Grace period (T_TALK) elapses, then billing starts. Tariff rate looked up from PH_ENGINE. |
| ON_HOOK | Handset replaced. Billing stops. Cost calculated with ceiling/rounding. Receipt written to RX.DAT/RX.IDX. Print spooler queues a print job. |
| LOCKED | Post-hangup lock period (T_LOCK). Booth returns to IDLE after expiry. |

## Abnormal Call Termination

If the call is abandoned before answer (T_ANSWER expires), no receipt is generated and no charge is incurred.

If communication errors (N_COM_ERR > MAX_COM_ERR) or dialing errors (N_DIAL_ERR > MAX_DIAL_ERR) exceed configured thresholds, an alarm is flagged in the view.

## Receipt Queues

- Receipts (MAX_RECEIPTS = 256). Holds completed, unpaid receipts. Used by the manual settlement window.

- PRNReceipts (MAX_PRN_RECEIPTS = 64). Holds receipts queued for reprinting or second-copy printing.

# Tariff Engine

## Call Classification

PH_ENGINE classifies each call into one of the following types:

| Type | Classification rule |
| --- | --- |
| Local | Matches a prefix in LOCAL.INF. |
| National DDN | Matches a DDN area code in DDN.INF. |
| International DDI | Matches a DDI country code in DDI.INF. |
| USA | Routed to USA destination; DDI tariff with USA-specific minimum minutes. |
| Border zone | Dialed with BORDER_ACCESS code. |
| Cellular | Dialed with CELLULAR_ACCESS code. |
| Special services | Dialed with SPECIAL_ACCESS code. Toll-free or special billing. |
| Telex | National telex rate via NA_TLX_BASE. |

## Cost Calculation

At call termination, cost is computed in the following steps:

- Duration. Raw elapsed seconds from answer signal to hangup (minus T_TALK grace period).

- Ceiling. Duration rounded up to the nearest CEIL_* fraction (1.0 = whole minute, 0.5 = half minute).

- Minimum minutes. If ceiled duration < MIN_, billed duration is promoted to MIN_.

- Rate lookup. Per-minute rate selected from tariff band based on call type, current hour, day type, and schedule state.

- Subtotal. Billed minutes × per-minute rate.

- Tax. TAX_PERCENT (or DDN_TAX / DDI_TAX) applied to subtotal.

- Rounding. Final total rounded to nearest M_ROUND unit.

# Receipt Formats and Printer Drivers

## Printer Driver DLLs

| DLL | FORM value | Description |
| --- | --- | --- |
| pr_dr80.dll | DR_80 | 80-column parallel double receipt |
| pr_drpre.dll | DR_PRE | 80-column pre-printed double receipt |
| pr_lin80.dll | LINEAL_80 | 80-column line printer (linear) |
| pr_sr80.dll | SR_80 | 80-column serial single receipt |
| pr_dr40.dll | DR_40 | 40-column parallel dual roll |
| pr_sr40.dll | SR_40 | 40-column serial single roll |
| pr_dr18.dll | DR_18 | 18-column parallel dual roll |
| pr_sr28.dll | SR_28 | 28-column serial single roll |
| pr_dreme.dll | DR_EME | 80-column EMETEL thermal |
| pr_drhal.dll | DR_HALF | 80-column half-page thermal |

## Receipt Content

Each receipt includes:

- Header: company name, city, operator name, NIT/tax ID, receipt number, date, time.

- Call detail: booth identifier/name, call type, destination, duration, per-minute rate.

- Financial block: subtotal, tax name and amount, total, rounding adjustment.

- Footer: configurable P_FOOTER lines.

# Transaction Database

## Record Structure

Each receipt record stored in RX.DAT includes:

- Receipt tag (TEL for telephony, EXT for extension).

- Booth number.

- Date and time of call start.

- Duration (seconds).

- Call type (local, DDN, DDI, cellular, border, special).

- Destination identifier (city/country name).

- Subtotal, tax amount, total cost.

- Receipt serial number.

- Paid / not-paid / toll-free flags.

## Statistics

DB_STATISTICS accumulates running totals from all records. Summaries are available at daily, weekly, monthly, and annual granularities. The TURN system supports multiple work shifts per day, each with its own statistics snapshot.

# Password Levels

| Level | Internal name | Access granted |
| --- | --- | --- |
| Backdoor | BACKDOOR | Full system access including configuration changes. Used by service technicians. |
| Supervisor | SUPERVISOR | Full operational access: configuration, reports, end-of-turn, telephony database. |
| User 1 | USER1 | Standard operator access: booth monitoring, printing receipts, manual settlement. |
| User 2 | USER2 | Restricted operator access. |
| Operator | OPERATOR | View-only access to extension services and information screens. Cannot modify configuration. |
| Utility | UTIL | Access for the UTIL diagnostic utility program. |

Passwords are 8 characters maximum and are stored encrypted in ST.CFG. The STM2 EEPROM validates the logged-in state; if a bad shutdown is detected, the system prompts for recovery confirmation before proceeding.

# System Log Events

| Event constant | Description |
| --- | --- |
| NORMALSTART | Application started successfully. |
| NORMALSHUTDOWN | Application exited cleanly. |
| APPBADTRY | Application failed to start (RES.DAT missing or corrupt). |
| CFGBADTRY | Application failed to start (ST.CFG missing or corrupt). |
| STM2BADTRY | Access denied: STM2 hardware authentication failed. |
| DONGLEBADTRY | Access denied: hardware dongle not found (demo build). |
| EEPROMBADTRY | Access denied: EEPROM version mismatch. |
| STM2GARBAGE | STM2 state was corrupted at startup; state was reset. |
| STM2BADSHUTDOWN | Previous session did not terminate cleanly; recovery initiated. |
| STM2RECOVER | System entered recovery mode after bad shutdown. |
| STM2IGNORERECOVER | Operator bypassed recovery after bad shutdown. |

Log entries include date, time, and the relevant event code. The log file is read-only when not being written; the application temporarily sets it read-write before appending.

# Error Messages

## Startup Errors

- “Acceso Negado” (Access Denied). STM2 hardware authentication failed, EEPROM version mismatch, or dongle not found. Application exits after this message.

- “Configuración no disponible” (Configuration not available). ST.CFG is missing or corrupted. Run the SETUP utility to reinstall.

- errno + “\RES.DAT”. Zinc resource file cannot be opened. Verify RES.DAT is present in the application directory.

- “Versión no válida” (Invalid version). EEPROM version does not match this release. Contact the distributor for a firmware update.

## Runtime Errors

- “El sistema está procesando información” (System is processing). Displayed in the exit dialog when the real-time engine has active calls. Wait for all booths to go idle before exiting.

- Out of memory. The new_handler is invoked when heap allocation fails. The system attempts to release the emergency memory reserve (2 KB) and display a message.

- GPF (General Protection Fault). Unhandled GPFs in 286 protected mode are caught by a custom exception handler (NewGPFHandler). The handler records the fault and attempts a controlled exit.

# Hardware Interface

## STB-x Cluster Board

Each cluster board handles 8 telephone lines via the ISA I/O port bus. Cluster boards are assigned consecutive 16-byte I/O blocks starting at APP_PORT_BASE. With 4 clusters, total I/O space used is 64 bytes.

The real-time engine polls cluster ports at regular timer ISR intervals. The PO_GENERAL output port drives the cash drawer relay (GP_CASH bit) when ACTIVATE_RELAY is set.

## EEPROM Dongle

The hardware protection dongle connects to the parallel port (LPT1). It contains an EEPROM storing the system serial number, the maximum allowed cluster count, and the version key. At startup, the application reads the EEPROM, validates the version, and records the login event. On normal exit, it records the logout. An abnormal exit leaves the STM2 state as BAD_SHUTDOWN, detected on the next startup.

## Booth Display

An optional customer-facing LED display can be connected to a serial port (DISPLAY_COM). The BoothDisplay class sends the current call cost or DISPLAY_DEFAULT_MESSAGE to the display in real time. Enabled by setting DISPLAY_ENABLE = TRUE in ST.INI.

SmartTar Reference Manual — Version 2.34 MicroDiseño Ltda. — Copyright © 1993–2003. All rights reserved. Gonzalo Contento Castañ